Rotary Hammer

RH720

GB

GB

Read this operators guide carefully, before using the machine. Ensure that you know how the machine works, and how it should be operated. Maintain the machine in accordance with the instructions, and make certain that the machine functions correctly. Keep this operator’s guide and other enclosed documentation with the machine.

TECHNICAL SPECIFICATIONS

| Model | RH720 |
| --- | --- |
| Rated Voltage | 220-240V ~ 50/60HZ |
| Power | 720W |
| No load speed | 0-1100/min |
| impact frequency | 0-4800/min |
| Impact energy | 2.8J |
| Drill bits | SDS plus |
| Max drill capacity | Concrete:24mm wood:30mm steel:13mm |
| Net weight | 3kg |

Over view

- Retaining bush

- Depth guide ruler

- Auxiliary handle

- Function selection knob

- Direction switch

- Switch trigger

- Lock button

- Locking button of function selection knob

General Safety Instructions for Electric Tools

This device is exclusively for private use! It is not fit for commercial use!

WARNING! Read all instructions. Failure to follow all instructions listed below may result in electric shock, fire and/or serious injury:

SAVE THESE INSTRUCTIONS

1)Work area

• Keep work area clean and well lit. Cluttered and dark areas invite accidents

• Do not operate machines in explosive atmospheres, such as in the presence of flammable liquids, gases or dust. Machines create sparks which may ignite the dust or fumes.

• Keep children and bystanders away while operating a machine. Distractions can cause you to lose control.

2)Electrical safety

• Machine plugs must match the outlet. Never modify the plug in any way. Do not use any adapter plugs with earthed (grounded) machines. Unmodified plugs and matching outlets will reduce risk of electric shock.

• Avoid body contact with earthed or grounded surfaces such as pipes, radiators, ranges and refrigerators. There is an increased risk of electric shock if your body is earthed or grounded.

• Do not expose machines to rain or wet conditions. Water entering a machine will increase the risk of electric shock.

• Do not abuse the cord. Never use the cord for carrying, pulling or unplugging the machine. Keep cord away from heat, oil, sharp edges or moving parts. Damaged or entangled cords increase the risk of electric shock.

• When operating a machine outdoors, use an extension cord suitable for outdoor use. Use of a cord suitable for outdoor use reduces the risk of electric shock.

3)Personal safety

• Stay alert, Watch what you are doing and use common sense when operating a machine. Do not use a machine while you are tired or under the influence of drugs, alcohol or medication. A moment of inattention while operating machines may result in serious personal injury.

• Use safety equipment. Always wear eye protection. Safety equipment such as dust mark, non-skid  safety shoes, hard hat, or hearing protection used for appropriate conditions will reduce personal injuries.

•Avoid accidental starting. Ensure the switch is in the off position before plugging in. Carrying machines with your finger on the switch or plugging in machines that have the switch on invites accidents.

•Remove any adjusting key or wrench before turning the machine on. A wrench or a key left attached to a rotating part of the machine may result in personal injury.

•Do not overreach. Keep proper footing and balance at all times. This enables better control of the machine in unexpected situations.

• Dress properly. Do not wear loose clothing or jewellery. Keep your hair, clothing and gloves away from moving parts. Loose clothes, jewellery or long hair can be caught in moving parts.

• If devices are provided for the connection of dust extraction and collection facilities, ensure these are connected and properly used. Use of these devices can reduce dust related hazards.

4)Machine use and care

• Do not force the machine. Use the correct machine for your application. The correct machine will do the job better and safer at the rate for which it was designed.

•Do not use the machine if the switch does not turn it on and off. Any machine that can not be controlled with the switch is dangerous and must be repaired.

•Disconnect the plug from the power source before making any adjustments, changing accessories, or storing machines. Such preventive safety measures reduce the risk of starting the machine accidentally.

• Store idle machines out of the reach children and do not allow persons unfamiliar with the machine or these instructions to operate the machine. Machines are dangerous in the hands of untrained users.

• Maintain machines. Check for misalignment or binding of moving parts, breakage of parts and any other condition that may affect the machines operation. If damaged, have the machine repaired before use. Many accidents are caused by poorly maintained machines.

•Keep cutting tools sharp and clean. Properly maintained cutting tools with sharp cutting edges are less likely to bind and are easier to control.

• Use the machine, accessories and tool bits etc., in accordance with these instructions and in the manner intended for the particular type of machine, taking into account the working conditions and the work to be performed. Use of the machine for operations different from intended could result in a hazardous situation.

5) Service

• Have your machine serviced by a qualified repair person using only identical replacement parts. This will ensure that the safety of the machine is maintained.

##### SPECIFIC SAFETY RULES FOR ROTARY HAMMER:

- Wear ear protectors. Exposure to noise can cause hearing loss.

- Use auxiliary handles supplied with the tool. Loss of control can cause personal injury.

- Hold power tools by insulated gripping surfaces when performing an operation where the cutting tool may contact hidden wiring or its own cord. Contact with a "live" wire will make exposed metal parts of the tool "live" and shock the operator.

- Wear a hard hat (safety helmet), safety glasses and/or face shield. Ordinary eye or sun glasses are NOT safety glasses. It is also highly recommended that you wear a dust mask and thickly padded gloves.

- Be sure the bit is secured in place before operation.

- Under normal operation, the tool is designed to produce vibration. The screws can come loose easily, causing a breakdown or accident. Check tightness of screws carefully before operation.

- In cold weather or when the tool has not been used for a long time, let the tool warm up for a while by operating it under no load. This will loosen up the lubrication. Without proper warm-up, hammering operation is difficult.

- Always be sure you have a firm footing. Be sure no one is below when using the tool in high locations.

- Hold the tool firmly with both hands.

- Keep hands away from moving parts.

- Do not leave the tool running. Operate the tool only when hand-held.

- Do not point the tool at any one in the area when operating. The bit could fly out and injure someone seriously.

- Do not touch the bit or parts close to the bit immediately after operation; they may be extremely hot and could burn your skin. Some material contains chemicals which may be toxic. Take caution to prevent dust inhalation and skin contact. Follow material supplier safety data.

OPERATION INSTRUCTIONS

Switch action

To start the tool, simply press the switch trigger (6).

Tool speed is increased by pressing further on the switch trigger. Release the switch trigger to stop.

For continuous operation, pull the switch trigger and then push in the lock button (7).

To stop the tool from the locked position, push the switch trigger fully, then release it.

Reversing switch action

If you need the drill turn with forward, turn the direction switch(5) to “”, if you need the drill turn with reverse, turn the direction switch (5) to “”.

CAUTION:

• Always check the direction of rotation before operation.

• Use the reversing switch only after the tool comes to a complete stop.

Changing the direction of rotation before the tool stops may damage the tool.

Selecting the action mode

This rotary hammer has four functions: hammer/drill/ hammer+drill, and drill bits angle adjustment. If you need select function, you should press the Locking button of function selection knob (8), then turn the function selection knob (4). If you need “hammer” function,

You should turn it to “”, if need ”drill” function, should turn it to “”, if need” hammer+drill” function, should turn to “” if need adjust drill bits angle, should turn to “”.

Torque limiter

The torque limiter will actuate when a certain torque level is reached. The motor will disengage from the output shaft. When this happens, the bit will stop turning.

Fix and take down drill bits

To fix the drill bits, just insert the drill bits in the hammer. To take down the drill bits, push the Retaining bush (1) to back, then pull out the drill bits.

HOW TO USE THE ROTARY HAMMER

Before setting up, repair or maintenance of the appliance you must always turn off the operating switch and pull out the mains plug!

Hammer drilling operation

Set the action mode changing knob to the symbol. Position the bit at the desired location for the hole, then pull the switch trigger. Do not force the tool. Light pressure gives best results. Keep the tool in position and prevent it from slipping away from the hole. Do not apply more pressure when the hole becomes clogged with chips or particles. Instead, run the tool at an idle, then remove the bit partially from the hole. By repeating this several times, the hole will be cleaned out and normal drilling may be resumed.

Chipping/Scaling

Set the action mode changing knob to the symbol. Hold the tool firmly with both hands. Turn the tool on and apply slight pressure on the tool so that the tool will not bounce around, uncontrolled. Pressing very hard on the tool will not increase the efficiency.

Drilling in wood or metal

Use the quick change drill chuck as standard equipment .Hold the ring and turn the sleeve counterclockwise to open the chuck jaws. Place the bit in the chuck as far as it will go. Hold the ring firmly and turn the sleeve clockwise to tighten the chuck. To remove the bit, hold the ring and turn the sleeve counterclockwise. Set the action mode changing knob to the symbol.You can drill up to 13mm diameter in metal and up to 30mm diameter in wood.

# MAINTENANCE

•Always be sure that the tool is switched off and unplugged before attempting to perform inspection or maintenance.

To maintain product SAFETY and RELIABILITY, repairs, carbon brush inspection and replacement, any other maintenance or adjustment should be performed by Authorized or Factory Service Centers, always using our replacement parts.

# ENVIRONMENTAL PROTECTION

Electrical products should not be disposed of with household waste.

Please recycle where facilities exist. Check with your local authority or retailer for recycling advice.

RH720 exploded view

RH720 part list